| UNIT-I |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BASICS OF MECHANISMS |  |  |  |  |  |  |  |
| PART – A |  |  |  |  |  |  |  |
| 1 | Write the different types of kinematic pairs based on the relative motion between them. | AU-ND 2019 | Remember BT-L1 |  |  | CO1 | PI 1.3.1 |
|  | - Based on nature of contact between elements: (a) Lower pair (b) Higher pair - Based on relative motion between pairing elements (a) Siding pair (b) Turning pair (revolute pair (c) Cylindrical pair - Based on the nature of mechanical constraint (a) Closed pair (b) Unclosed or force closed pair |  |  |  |  |  |  |
| 2 | Differentiate machine and structure. | AU-ND 2019 | Remember BT-L1 |  |  | CO1 | PI 1.2.1 |
|  | Machine; Structure. Machine is a mechanism or collection of mechanism which transmits force from the source of power to the resistance (load) to overcome and thus performs useful mechanical work.; Structure is the assemblage of resistant bodies without any relative motion between the links. The link of machine made transmit both power relative motion and forces.; The member of the structure transmits forces only.. E.g IC engine; E.g Bridges. |
| 3 | Define rigid link and give examples. | AU-AM 2019 |  | Understand BT-L2 |  | CO1 | PI 1.3.1 |
|  | A rigid link is one that does not undergo any deformation while transmitting motion. They are considered rigid if they do not undergo appreciable deformation while transmitting motion. For Example, crank and connecting rod |  |  |  |  |  |  |
| 4 | List the inversions of a double slider crank chain. | AU-AM 2019 | Remember BT-L1 |  |  | CO1 | PI 1.3.1 |
|  | They are three important inversions of double slider crank chain. - Elliptical trammel. - Scotch yoke mechanism. - Oldham's Coupling |  |  |  |  |  |  |
| 5 | Sketch a slider crank mechanism indicating their input and output motions. |  | Create BT-L6 |  |  | CO1 | PI 1.3.1 |
| 6 | Define the Grubler’s criterion for plane mechanism with mathematical expressions. |  | Remember BT-L1 |  |  | CO1 | PI 2.1.3 |
|  | DOF = 3 (n – 1) – 2 l – h Where, DOF = Degrees of freedom of the mechanism, n = Number of links, l = Number of lower pairs ( or binary joints), and h = Number of higher pairs. |  |  |  |  |  |  |
| 7 | Name any two inversions of single slider crank chain. |  |  |  | Remember BT-L1 | CO1 | PI 1.2.1 |
|  | The inversions of single slider crank chain are - First inversion. Application- Reciprocating engine, reciprocating compressor etc... - Second inversion. Application- Whitworth quick return mechanism etc... - Third inversion. Application- Slotted crank mechanism, Oscillatory engine etc.., |  |  |  |  |  |  |
| 8 | What type of kinematic pair exists between human shoulder and arm based on nature of contact and nature of relative motion? |  |  |  | Understand BT-L2 | CO1 | PI 1.3.1 |
|  | Based on nature of contact: - Lower pair - Higher pair Based on relative motion: - Sliding pair - Turning pair - Rolling pair &Screw or Helical pair |  |  |  |  |  |  |
| 9 | Sketch and define Transmission angle of a four bar mechanism. |  |  |  | Create BT-L6 | CO1 | PI 1.4.1 |
|  | The transmission angle reaches its extreme when the driving crank and the fixed link are collinear as shown in Figure. The extreme value μ min will occur in either of the two positions. It is common practice to calculate both values. |  |  |  |  |  |  |
| 10 | Classify kinematic pairs based on nature of contact. Give examples. |  |  |  | Remember BT-L1 | CO1 | PI 1.2.1 |
|  | The classification of kinematic pair based on nature of contact are - Lower pair: Examples include nut and bolt, ball and socket joints, shaft rotating in bearing, etc. - Higher pair: Examples include cam and follower, roller bearings, ball bearings, etc |  |  |  |  |  |  |
| 11 | When a linkage becomes mechanism? |  |  |  | Remember BT-L1 | CO1 | PI 1.3.1 |
|  | A linkage becomes a mechanism when one of the links is defined as the fixed link. The mechanism is an assemblage of links used to transmit and modify motion (without considering forces involved). |  |  |  |  |  |  |
| 12 | What is an inversion? |  |  |  | Remember BT-L1 | CO1 | PI 1.2.1 |
|  | The method of obtaining different mechanisms by fixing different links in a kinematic chain is known as inversion of mechanism. The kinematic inversion of four bar chain is classified as beam engine, compelled locomotive |  |  |  |  |  |  |

| 13 | Define transmission angle and what will happen to the mechanism if it is equal to zero? |  |  | Understand BT-L2 | CO1 | PI 1.2.1 |
| --- | --- | --- | --- | --- | --- | --- |
|  | The angle γ between the coupler and the follower is called the transmission angle. When the angle becomes zero, the mechanical advantage becomes infinite and the linkage is said to be in a toggle position. |  |  |  |  |  |
| 14 | Differentiate Ackermann steering gear and Davis steering gear mechanism. |  |  | Understand BT-L2 | CO1 | PI 1.3.1 |
|  | Ackermann Steering Gear; Davis Steering Gear. The whole mechanism of the Ackermann steering gear is on back of the front wheels.; In Davis steering gear, it is in front of the wheels.. The Ackermann steering gear consists of turning pairs.; Davis steering gear consists of sliding members.. |
| 15 | What is Kutzbach’s criterion for degree of freedom? |  |  | Remember BT-L1 | CO1 | PI 2.1.3 |
|  | The number of degrees of freedom of a plane mechanism can be determined by using Kutzbach criterion is “n = 3(l-1)-2j-h”. Where, n = degrees of freedom, j = number of binary jointsh = number of higher pair |  |  |  |  |  |
| 16 | Define mechanical advantage |  |  | Remember BT-L1 | CO1 | PI 1.4.1 |
|  | The mechanical advantage is defined as the ratio of output torque to the input torque. It is also defined as the ratio of the load to the effort. Mechanical advantage = (TB/TA) |  |  |  |  |  |
| 17 | What is meant by degree of freedom? |  |  | Remember BT-L1 | CO1 | PI 2.1.2 |
|  | The degree of freedom of a mechanism is the number of independent parameters required to specify the location of every link with in the mechanism. (or) Degree of freedom is defined as the number of independent co ordinate required to specify the relative motion so it is known as degree of freedom and its also called us mobility. |  |  |  |  |  |
| 18 | State any four inversions of a single slider crank chain. |  |  | Remember BT-L1 | CO1 | PI 1.3.1 |
|  | The inversions of a single slider crank chain as follows - Reciprocating engine - Reciprocating compressor - Rotary engine Hand pump |  |  |  |  |  |
| 19 | Name any two applications of four bar chain. |  |  | Remember BT-L1 | CO1 | PI 1.3.1 |
|  | The applications of four bar chain as follows - Beam engine. - All rotary oscillating converters. - Coupling rod of a locomotive. - Watt’s indicator mechanism. |  |  |  |  |  |
| 20 | Write Grashof’s law for four bar mechanism. |  |  | Understand BT-L2 | CO1 | PI 1.3.1 |
|  | Grashof’s law states that the sum of the shortest and longest links cannot be greater than the sum of the remaining two links length, if there is to be continuous relative motion between two members. |  |  |  |  |  |
| 21 | Define completely constrain motion. |  |  | Remember BT-L1 | CO1 | PI 1.3.1 |
|  | Completely constrained motion: When the motion between a pair is limited to a definite direction irrespective of the direction of force applied, then the motion is said to be a completely constrained motion. |  |  |  |  |  |
| 22 | Define Pantograph. |  |  | Remember BT-L1 | CO1 | PI 1.3.1 |
|  | A pantograph is a mechanism used to produce paths on an enlarged or reduced scale as exactly as possible the path described by a given point. It is based on four bar kinematic chain mechanism and its applications. |  |  |  |  |  |
| 23 | Differentiate between Machine and Mechanism. |  |  | Understand BT-L2 | CO1 | PI 1.4.1 |
|  | Machine; Mechanism. Machine is a mechanism or collection of mechanism which transmits force from the source of power to the resistance (load) to overcome and thus performs useful mechanical work. E.g Lathe,Shaper; Combination of rigid or resistant bodies connected that they move upon each other with definite relative motion E.g I.C Engine. |
| 24 | List out types of joints used in chains |  |  | Remember BT-L1 | CO1 | PI 1.2.1 |
|  | Types of joints used in chains are - Binary joint - Ternary joint - Quaternary joint |  |  |  |  |  |
| 25 | Define mechanism and list out the mechanisms. |  |  | Remember BT-L1 | CO1 | PI 1.4.1 |
|  | When one of the links of a kinematic chain is fixed, the chain is known as Mechanism. It may be used for transmitting or transforming motion e.g. engine indicators, typewriter etc., A mechanism with four links is known as simple mechanism, and the mechanism with more than four links is known as compound mechanism. |  |  |  |  |  |
| PART – B |  |  |  |  |  |  |
| 1 |  | State whether the following links shown in fig. (a, b, c and e) are mechanisms with one degree of freedom. If not make suitable changes but the number of links should not be varied more than one | AU-ND 2019 | Apply BT-L3 | CO1 | PI 2.4.1 |

| 2 | For kinematic linkage shown in Fig i) Calculate the total number of binary, ternary and quaternary links (ii) total number of links (iii) total number of joints or pairs (iv) the number of degrees of freedom. Comment on the kinematic linkage based on mobility. | AU-ND 2019 | Apply BT-L3 | CO1 | PI 2.1.3 |
| --- | --- | --- | --- | --- | --- |
| 3 | Explain how are the Whitworth quick return mechanism and Crank and slotted lever mechanism different from each other | AU-ND 2019 | Apply BT-L3 | CO1 | PI 1.4.1 |
| 4 | Explain with neat sketch, kinematic inversions of four bar chain. | AU-AM 2019 | Remember BT-L1 | CO1 | PI 1.4.1 |
| 5 | With neat -sketch, explain the crank and slotted ·1ever quick return Mechanism. | AU-AM 2019 | Remember BT-L1 | CO1 | PI 1.2.1 |
| 6 | - State and brief the Kutzbach criterion for planar mechanisms and using this criterion, determine the arrangements shown in Fig as a structure or a constrained mechanism or un constrained mechanism |  | Analyze BT-L4 | CO1 | PI 2.2.3 |
|  | ii) Define transmission angle of a four bar mechanism and explain its significance. Also neatly sketch a crank rocker mechanism in its minimum and maximum transmission angle positions. |  | Apply BT-L3 | CO1 | PI 1.2.1 |
| 7 | Define kinematic inversion and neatly sketch an elliptical trammel, i.e., one of the inversion of a double slider crank chain. Also prove or disprove that all the points on the revolving link of the elliptic trammel will trace ellipses only |  | Remember BT-L1 | CO1 | PI 2.3.2 |
|  | Sketch and brief peaucellier exact straight line mechanism. |  | Remember BT-L1 | CO1 | PI 2.1.1 |
| 8 | Write in detail with neat sketch, any three inversion of a double slider crank chain. |  | Create BT-L6 | CO1 | PI 1.2.1 |
| 9 | Describe with neat sketch, the mechanism obtained by the inversion of four - bar chain. |  | Create BT-L6 | CO1 | PI 1.2.1 |
| 10 | ABCD is a four bar chain with link AD fixed. The length of the links AB = 190 mm, BC = 280 mm, CD = 280 mm, and AD = 500 mm. The crank AB has an angular velocity of 10 rad/s clockwise. Draw the velocity and acceleration diagram when angle BAD = 550 and B and C lie on same side of AD. Find the acceleration and angular acceleration of the links BC and CD. |  | Create BT-L6 | CO1 | PI 1.2.1 |
| 11 | What is a kinematic inversion? Discuss any three applications of inversions of slider crank mechanism with suitable sketches. |  | Remember BT-L1 | CO1 | PI 1.2.1 |
| 12 | - Find the degrees of freedom for the mechanisms shown in below figure. |  | Analyze BT-L4 | CO1 | PI 1.2.1 |
|  | ii) Explain the mechanical advantage and transmission angle related to four bar mechanism. |  | Understand BT-L2 | CO1 | PI 1.2.1 |
| 13 | Figure shows a mechanical press used to exert large forces to insert a small part into a larger one. Draw a kinematic diagram, using the end of the handle as a point of intersect. Also compute the degrees of freedom. |  | Remember BT-L1 | CO1 | PI 1.2.1 |
| 14 | Classify kinematic pairs based on degrees of freedom. |  | Remember BT-L1 | CO1 | PI 1.2.1 |
|  | What is inversion and list its properties? |  | Remember BT-L1 | CO1 | PI 1.2.1 |
| 15 | - Find the degrees of freedom for the following figures. |  | Evaluate BT-L5 | CO1 | PI 1.2.1 |
|  | State the inconsistencies of Grubler’s criterion. |  | Remember BT-L1 | CO1 | PI 1.2.1 |
| 16 | Sketch and explain the mechanism obtained by double slider crank mechanism. |  | Create BT-L6 | CO1 | PI 1.2.1 |
| 17 | Explain the following mechanism with neat sketches: - Ratchet and Escapement mechanism. - Indexing mechanism. - Elliptical trammel - Scotch yoke mechanism. - Quick return mechanism |  | Remember BT-L1 | CO1 | PI 1.3.1 |
| 18 | Explain with a neat sketch, how an offset slider crank mechanism can be used as a quick-return motion mechanism? Derive an expression to find the quick-return ratio. |  | Remember BT-L1 | CO1 | PI 2.1.3 |
| 19 | Sketch and explain the four inversions of single slider crank chain. |  | Remember BT-L1 | CO1 | PI 1.2.1 |
| 20 | What are straight line mechanisms? Sketch the Peaucellier straight line motion mechanism and prove that the generating point moves in straight line. |  | Remember BT-L1 | CO1 | PI 1.2.1 |
| PART – C |  |  |  |  |  |
| 1 | i) Identify the type of inversion of the four bar mechanism shown in Fig stating the reasons for your answer. The figure indicates the dimensions 1n standard units of length is a = 1 unit b = d= 3 units c = 2 units. ii) Find the· maximum. and minimum transmission angles for the same mechanism graphically and analytically and compare them. | AU-ND 2019 | Apply BT-L3 | CO1 | PI 1.3.1 |
| 2 | In four bar crank-rocker mechanism, the forward motion of the rocker take place during the 2250 cionstant speed rotation of the crank. Determine the time –ratio and prove that this mechanism can be used as quick-return mechanism. | AU-AM 2019 | Apply BT-L3 | CO1 | PI 1.3.1 |
| 3 | In order to form a four bar kinematic chain, state and prove the assembly condition of link lengths by selecting suitable link lengths which are within the range of 35 mm to 175 mm and also, satisfying Grashof’s law. | AU-AM 2019 | Apply BT-L3 | CO1 | PI 1.3.1 |
| UNIT-II KINEMATICS OF LINKAGE MECHANISMS PART – A |  |  |  |  |  |
| 1 | What are the types of instantaneous centres? | AU-ND 2019 | Remember BT-L1 | CO2 | PI 1.4.1 |
|  | The first two types i.e. fixed and permanent instantaneous centres are together known as primary instantaneous centres. The third type is known as secondary instantaneous centres. |  |  |  |  |
| 2 | Define rubbing velocity at a pin joint. | AU-ND 2019 | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | Rubbing velocity at a pin joint It is defined as the algebraic sum between the angular velocities of the two links which are connected by pin joints, multiplied by the radius of pin |  |  |  |  |
| 3 | What is meant by normal component of acceleration? | AU-AM 2019 | Remember BT-L1 | CO2 | PI 1.3.1 |
|  | The tangential acceleration is a measure of the rate of change in the magnitude of the velocity vector, i.e. speed, and the normal acceleration are a measure of the rate of change of the direction of the velocity vector |  |  |  |  |
| 4 | Define Centroid. | AU-AM 2019 | Understand BT-L2 | CO2 | PI 1.2.1 |
|  | Centroid, in kinematics, is the path traced by the instantaneous center of rotation of a rigid plane figure moving in a plane. There are two types of centroids: a space Centroid, and a body centroid. |  |  |  |  |
| 5 | How will you find out the total acceleration from its normal and tangential components? |  | Analyze BT-L4 | CO2 | PI 1.4.1 |
|  | Consider two points A and B on a rigid link as shownin Fig. Let the point B moves with respect to A, with an angular velocity of ω rad/s and let α rad/s2 be the angular acceleration of the link AB. |  |  |  |  |
| 6 | Mention any two motives for doing acceleration analysis of mechanism or machines. |  | Remember BT-L1 | CO2 | PI 1.3.1 |
|  | The main motives for doing acceleration analysis of mechanism or machines are - To find the acceleration of the links used in mechanism. - To find the links form a mechanism or a truss |  |  |  |  |
| 7 | Define Coriolis component of acceleration. |  | Remember BT-L1 | CO2 | PI 1.3.1 |
|  | Coriolis components of acceleration is states that the distance between the two points is not fixed, then the total acceleration will have one additional acceleration component which is known as Coriolis component. |  |  |  |  |
| 8 | State the Arnold Kennedy theorem. |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | It states that if three bodies move relatively to each other, they have instantaneous centers and lie on a straight line. This theorem is used for the locating instantaneous centre will become laborious when the mechanism become complicated and the more number links .this theorem is simpler and practical way locating the instantaneous centre |  |  |  |  |
| 9 | Illustrate the instantaneous centers of a typical four bar mechanism. |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | The types of instantaneous centres are followed - Fixed instantaneous centres - Permanent instantaneous centres - Neither fixed nor permanent instantaneous centres |  |  |  |  |
| 10 | What is a relative pole, with respect to velocity analysis? |  | Remember BT-L1 | CO2 | PI 1.4.1 |
|  | A pole of the moving link is the centre of its rotation with respect to a fixed link. However, if the rotation of the link is considered relative to another moving link, the pole is known as the relative pole. |  |  |  |  |
| 11 | What are the different methods used for finding the velocity? |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | The different methods used for finding the velocity are - Relative velocity method (Graphical method) - Instantaneous centre method (Graphical method) - Analytical method - Klien’s construction method (Graphical method) |  |  |  |  |
| 12 | What is meant by coincident points? |  | Understand BT-L2 | CO2 | PI 1.2.1 |
|  | The coincident point are defined as the when a point on one link is sliding along another rotating link, then the point is known as coincident point in other words ,coincident point is point on slider such that the slider slides along the another rotating link so it called coincident point. |  |  |  |  |
| 13 | What is the magnitude of linear velocity of a point B on a link AB relative to A? |  | Understand BT-L2 | CO2 | PI 1.4.1 |
|  | The magnitude of linear velocity of a point B on a link AB, which rotates with ‘ 𝛚’ angular velocity with respect to A is: VBA = 𝛚 AB *AB Where 𝛚 = angular velocity in rad/s |  |  |  |  |
| 14 | Define instantaneous centre. |  | Understand BT-L2 | CO2 | PI 1.3.1 |
|  | This combined motion of rotation and translation of the link may be assumed to be a motion of pure rotation about some centre is known as the instantaneous centre of rotation. It shows the balanced kinematic pair. |  |  |  |  |
| 15 | Define velocity and speed. |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | Velocity may be defined as the rate of change of displacement of a body with respect to the time so it is known as velocity it has both magnitude and direction ,therefore it is a vector quantity . Speed may be defined as the rate of change of linear displacement of a body with respect to the time. |  |  |  |  |
| 16 | Define acceleration in mechanisms. |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | The rate of change of velocity with respect to time is known as acceleration. If acceleration acts in the direction of change in velocity. Thus acceleration is also a vector quantity of machines and mechanism. The acceleration analysis plays a vital role in design. |  |  |  |  |
| 17 | What are properties of instantaneous centre? |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | A rigid link rotates instantaneously relative to another link at the instantaneous centre for the configurations of the mechanism. The two rigid links have no linear velocity relative to each other at the instantaneous centre. At this point the two rigid links have the same linear velocity relative to the rigid link |  |  |  |  |
| 18 | State the application of an offset slider-crank mechanism. |  | Apply BT-L3 | CO2 | PI 1.3.1 |
|  | The offset slider crank mechanism is used to get reciprocating motion of the slider and is also used as a quick return mechanism. With an offset slider-crank mechanism, an offset distance is introduced. This offset distance is referred to as L1 and is the fixed distance between the crank arm pivot point and the slider axis. |  |  |  |  |
| 19 | Explain how the direction of Coriolis acceleration is obtained. |  | Apply BT-L3 | CO2 | PI 1.3.1 |
|  | The direction of Coriolis component is the direction of relative velocity vector for the two coincident points rotated at 90 degree in the direction of angular velocity of rotation of link. This is happen when a point on one link is sliding along the rotating link, such as quick return mechanism. |  |  |  |  |

| 20 | Define rubbing velocity at a pin point. |  | Remember BT-L1 | CO2 | PI 1.4.1 |
| --- | --- | --- | --- | --- | --- |
|  | The rubbing velocity is defined as the algebraic sum between the angular velocities of the two links which are connected by pin joints, multiplied by the radius of the pin. Two links of a mechanism having turning point will be connected by pins. When the links are motion they rub against pin surface. The velocity of rubbing of pins depends on the angular velocity of links relative to each other as well as direction. |  |  |  |  |
| 21 | Name any four common mechanisms with specific applications. |  | Remember BT-L1 | CO2 | PI 2.2.4 |
| Sl. No.; Mechanisms; Applications. 1.; Crank and lever mechanism; Beam engine. 2.; Double crank mechanism; Coupling rod of a locomotive. 3.; Double rocker mechanism; Pantograph; Ackermann steering. 4.; Whitworth quick-return mechanism; Shaping and slotting machines.. |
| 22 | What are properties of instantaneous centre? |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | A rigid link rotates instantaneously relative to another link at the instantaneous centre for the configurations of the mechanism. The two rigid links have no linear velocity relative to each other at the instantaneous centre. At this point the two rigid links have the same linear velocity relative to the rigid link. |  |  |  |  |
| 23 | Define displacement. |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | It may be defined as the distance moved by a body with respect to a fixed certain fixed point. When there is no displacement in a body it is said to be at rest and when it is being displaced, it is said to be in motion. |  |  |  |  |
| 24 | What are the types of motions? |  | Remember BT-L1 | CO2 | PI 1.2.1 |
|  | The types of motion are followed: - Rectilinear motion. - Curvilinear motion. - Circular motion. |  |  |  |  |
| 25 | Define slider crank mechanism. |  | Remember BT-L1 | CO2 | PI 1.4.1 |
|  | Slider and crank mechanism is an arm attached at right angles to a rotating shaft by which reciprocating motion is imparted to or received from the shaft. It is used to convert circular motion into reciprocating motion, or vice versa. |  |  |  |  |
| PART – B |  |  |  |  |  |
| 1 | State and explain angular velocity ratio theorem. | AU-ND 2019 | Understand BT-L2 | CO2 | PI 1.2.1 |
| 2 | In a slider crank mechanism, the lengths of the crank and connecting rod are 200 mm and 800 mm respectively. Locate all the I-centers of the mechanism for the position of the crank when it has turned 30° from the inner dead centre. Also find the velocity of the slider and the angular velocity of the connecting rod if the crank rotates at 40 rad/s, clockwise. | AU-ND 2019 | Apply BT-L3 | CO2 | PI 1.4.1 |
| 3 | Fig. shows a mechanism in which OA =QC= 100 mm, AB= QB= 300 mm and CD = 250 mm. The crank OA rotates at 150 rpm in the clockwise direction. Determine the (i) velocity of the slider at D (ii) angular velocities of links QB and AB (iii) rubbing velocity at the pin B which is 40 mm in diameter. | AU-ND 2019 | Analyze BT-L4 | CO2 | PI 1.3.1 |
| 4 | i) State and prove the Kennedy’s theorem of three instantaneous centres. | AU-AM 2019 | Apply BT-L3 | CO2 | PI 1.2.1 |
|  | ii) Give the types of instantaneous centres with examples. | AU-AM 2019 | Apply BT-L3 | CO2 | PI 1.3.1 |
|  | iii) Give the procedure to be followed for locating instantaneous centres for four bar mechanisms. | AU-AM 2019 | Apply BT-L3 | CO2 | PI 1.2.1 |
| 5 | The crank of a slider crank mechanism rotates clockwise at a constant speed of 300 rpm. The crank is 150 mm and the connecting rod is 600 mm long. Determine. - Linear velocity and acceleration of the midpoint of the connecting rod, and - Angular velocity and angular acceleration of the connecting rod, at a crank angle of 45° from inner dead centre position. | AU-AM 2019 | Analyze BT-L4 |  | PI 1.3.1 |
| 6 | A four bar mechanism ABCD has the following lengths: Fixed link, AB-60 mm; Input link,AoA-30 mm; Coupler, AB-45 mm; Output link, BC-50 mm. Pivot A is left of pivot B and both in pin joints A and B are above the horizontal fixed link. A point C is on the straight extension of the coupler, such that BC=25mm. Input link rotates at a constant speed of 20 rpm clockwise. Determine the linear velocities of points B and C separately, and angular velocities of the coupler and output link, when the link is 600 counter clockwise from fixed link. |  | Create BT-L6 | CO2 | PI 1.2.1 |
| 7 | What is kinematic synthesis? Name the three phases of kinematic synthesis and classify the linkage synthesis problems. |  | Remember BT-L1 | CO2 | PI 1.4.1 |
| 8 | State and prove Aronhold-Kennedy theorem related to instantaneous centers. |  | Remember BT-L1 | CO2 | PI 1.2.1 |
| 9 | Explain in detail, the concept of Coriolis component of acceleration with neat sketches and equations. |  | Remember BT-L1 | CO2 | PI 1.2.1 |
| 10 | In a crank and slotted lever quick return motion mechanism, the distance between the fixed centres is 240 mm and the length of the driving crank is 120 mm. Find the inclination of the slotted bar with the vertical in the extreme position and the time ratio of cutting stroke. If the length of the slotted bar is 450 mm, find the length of the stroke if the line of stroke passes through the extreme position of the free end of the lever. |  | Create BT-L6 | CO2 | PI 1.4.1 |
| 11 | Locate all the instantaneous centres of the slider crank mechanism. The crank (OA) is 160 mm and the connecting rod (AB) is 470 mm long. If the crank rotates clockwise with an angular velocity of 12 rad/s, Determine 1. Linear velocity of the slider (B) 2. Angular velocity of the connecting rod (AB), at a crank angle of 300 from the IDC position using instantaneous centre method. |  | Apply BT-L3 | CO2 | PI 1.4.1 |
| 12 | Design a cam for operating exhaust valve of an oil engine. It is required to give simple harmonic motion during opening of the valve with 1200 of cam rotation and simple harmonic motion during closing of the valve with 600 of the cam rotation. The valve must remain in the fully open position for 300 of cam rotation. The lift of the valve 50 mm and the least radius of the cam is 25 mm. The follower is provided with a lower radius of 10 mm and its line of stoke passes through the axis of the cam. |  | Create BT-L6 | CO2 | PI 2.1.2 |
| 13 | An engine mechanism is shown in figure. The crank CB = 200 mm and the connecting rod BA = 600 mm. In the position shown, the crankshaft has a speed of 560 rad/s and an angular acceleration of 800 rad/s2. Find: (i) angular velocity of AB and (ii) angular acceleration of AB. |  | Evaluate BT-L5 | CO2 | PI 1.1.2 |
| 14 | Locate all the instantaneous centres of the slider crank mechanism as shown in figure. The lengths of the OB and connecting rod AB are 100 mm and 400 mm respectively. If the crank rotates clockwise with an angular velocity of 10 rad/s, find: (i) Velocity of the slider A, and (ii) Angular velocity of the connecting rod AB. |  | Evaluate BT-L5 | CO2 | PI 1.1.2 |
| 15 | The diagram shows part of a quick return mechanism. P in A slides in the slot when the disc is rotated. Calculate the angular velocity and acceleration of link BC when Ѳ= 600 and ω = 100 rad/sec . |  | Evaluate BT-L5 | CO2 | PI 1.3.1 |

| 16 | The diagram shows part of the quick return mechanisms. The pin A slides in the slot when the disc is rotated. Calculate the angular velocity and acceleration of link BC when θ = 600 and ω = 100 rad/s. |  | Apply BT-L3 |  | CO2 | PI 1.3.1 |
| --- | --- | --- | --- | --- | --- | --- |
| 17 | Derive the expression for coriolis component of acceleration with neat sketch and give its direction for various conditions. |  | Apply BT-L3 |  | CO2 | PI 1.2.1 |
| 18 | In a simple steam engine, the lengths of the crank and the connecting rod are 100 mm and 400 mm respectively. The weight of the connecting rod is 50 kg and its centre of mass is 220 mm from the cross head centre. The radius of gyration about the centre of mass is 120 mm. If the engine speed is 300 rpm and the crank has turned 450 from IDC. Determine - The angular velocity and acceleration of the connecting rod and - (ii) Kinetic energy of the connecting rod. |  | Analyze BT-L4 |  | CO2 | PI 1.4.1 |
| 19 | PQRS is a four bar chain with a fixed link PS. The lengths of the links are: PQ=62.5 mm, QR=175 mm, RS=112.5 mm and PS= 200 mm. The crank PQ rotates at 10 rad/s clockwise. Draw the velocity and acceleration diagram when angle QPS=600 and find the angular velocity and angular acceleration of the links QR and RS.. |  | Create BT-L6 |  | CO2 | PI 1.4.1 |
| 20 | In a slider crank mechanism, the length of crank is 200 mm and length of connecting rod is 825 mm. The angular velocity and acceleration of crank is 60 rad/s and 1100 rad/s2. When the crank has turned 1200 from the inner dead centre, find (i) the velocity and acceleration of piston and (ii) angular velocity and acceleration of connecting rod. |  | Create BT-L6 |  | CO2 | PI 1.3.1 |
| 21 | Explain Coriolis component of acceleration and derive an expression for Coriolis component of acceleration. |  | Remember BT-L1 |  | CO2 | PI 2.1.2 |
| 22 | Explain Klein’s construction for slider crank mechanism when the crank rotates with uniform angular velocity. |  | Remember BT-L1 |  | CO2 | PI 2.1.2 |
| 23 | In a four bar mechanism ABCD, the lengths of various links are: fixed link AD=450 mm, input link AB=100 mm, coupler BC=400 mm, output link CD=175 mm. If the speed of the driver is 220 rpm clockwise, find the angular velocity and acceleration of the coupler and output link when the inclination of the output link is 600 with the fixed link. |  | Apply BT-L3 |  | CO2 | PI 1.4.1 |
| 24 | What is the importance of finding acceleration of various points in a mechanism? Name the two different methods of finding acceleration in a mechanism. What do you mean by Coriolis component of acceleration? When it will exist? Prove that the component of acceleration is equal to 2 ν ω, where ν is linear velocity of slider and ω is angular velocity of link. |  | Apply BT-L3 |  | CO2 | PI 1.4.1 |
| 25 | In a slider crank mechanism, the length of crank OB and connecting rod are 125 mm and 500 mm respectively. The centre of gravity G of the connecting rod is 275 mm from the slider A. The crank speed is 600 rpm clockwise. When the crank has turned 450 from the inner dead centre position, determine (i) Velocity of the slider A, (ii) Velocity of the point G and (iii) Angular velocity of the connecting rod AB. |  | Apply BT-L3 |  | CO2 | PI 1.4.1 |
| PART – C |  |  |  |  |  |  |
| 1 | The dimension of the mechanism as shown in the figure AB=0.45m ,BD=1.5m BC=CE=0.9m the crank AB turns uniformly at 180rpm in the clock wise direction and the block at D and E are working in frictionless guide. Draw the velocity diagram for the mechanism and find the velocity of the slides D and E in their guide.aso determine the turning moment at A if a forced of 50N at on the direction of arrow X and a force of 750N act on E in the direction of arrow Y. |  | Create BT-L6 | CO2 |  | PI 1.3.1 |
| 2 | The mechanism of a wrapping machine, as shown in fig, has the following dimensions OA = 100 mm; AC = 700 mm; BC = 200 mm; OC = 200 mm; OE = 400 mm; OD = 200 mm and BD = 150 mm. The crank OA rotates at a uniform speed of 100 rad/s. Find the velocity of the point E of the bell crank lever by instantaneous centre method. |  | Create BT-L6 | CO2 |  | PI 1.3.1 |
| UNIT-III KINEMATICS OF CAM MECHANISMS PART – A |  |  |  |  |  |  |
| 1 | Define prime circle and pitch curve of a cam. | AU-ND 2019 | Remember BT-L1 | CO3 |  | PI 1.3.1 |
|  | The prime circle is defined as the smallest circle which can be drawn tangent to the locus of the centerline of the follower. The locus of the centerline of the follower is called the pitch curve. All radial cams will have a base circle, regard- less of the follower type used |  |  |  |  |  |
| 2 | List the methods used to reduce the pressure angle of a cam. | AU-ND 2019 | Remember BT-L1 | CO3 |  | PI 1.3.1 |
|  | - Change the pivot-point of the cam-follower. - If the follower is a 'translating follower', - Move the axis of the follower so that it is not through the centre of the cam - Increase the size of the cam. The pressure-angle will decrease as you increase the size of the cam |  |  |  |  |  |
| 3 | Differentiate between radial cam and. cylindrical cam. | AU AM 2019 | Remember BT-L1 | CO3 |  | PI 1.2.1 |
|  | Disc or Radial Cams In these the working surface of the cam is shaped that the reciprocation or oscillation of the follower is in a plane at right angles to the axis of the cam. Cylindrical Cams These are often used in machine- tools and the cam imparts an oscillation or reciprocation to the follower in a plane parallel to the axis of the cam. |  |  |  |  |  |
| 4 | Draw the displacement diagram for, a follower when it moves with uniform acceleration and retardation. | AU-AM 2019 | Create BT-L6 | CO3 |  | PI 1.4.1 |
| 5 | Classify the translating cam followers based on their position. |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | According to translating cam followers. The followers, according to its motion, are of the following two types: - Reciprocating type follower - Rotating type follower |  |  |  |  |  |
| 6 | Sketch and name a specified contour cam. |  | Create BT-L6 | CO3 |  | PI 1.4.1 |
| 7 | Differentiate radial cam and cylindrical cams. |  | Understand BT-L2 | CO3 |  | PI 1.3.1 |
|  | Radial cam; Cylindrical cam. In this cam, the follower reciprocates (or) oscillates in a direction perpendicular to the axis.; In this the follower reciprocates (or) oscillates in a direction parallel to the cam axis.. Used in CNC machine.; Used in internal combustion engine.. |
| 8 | Name the cam follower extensively used in air-craft engines. |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | The cam follower generally used in air-craft engines is. - Knife edge follower. - Flat faced follower. - Spherical faced follower. - Roller follower.. |
| 9 | Which type of cam follower motion is preferred for high speed engines? Why? |  | Remember BT-L1 | CO3 |  | PI 1.2.1 |
|  | For high speed engines, the cam follower should move with Cycloidal motion. When the cam is rotating with high speed then the follower has ability to escape from above the cam to avoid it we used cycloidal follower in case of high speed. |  |  |  |  |  |
| 10 | Give any two applications of cam mechanism in IC engines. |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | The cam follower system used in an automotive valve actuation application. - To operate the rocker arm for fuel inlet and exhaust in IC engines. - To operate the Timing valve in IC engines |  |  |  |  |  |
| 11 | Define Cam. |  | Remember BT-L1 | CO3 |  | PI 1.3.1 |
|  | A cam is defined as a rotating mechanical member used for transmitting desired motion to a follower by direct contact so it is called as cam. Some examples of cam are - Radial or disc cams. - Cylindrical or barrel cams. - End or face cams & Wedge cams |  |  |  |  |  |
| 12 | What are the motions of cam follower? |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | The motion of cam follower are as follows, - Uniform motion. - Simple harmonic motion. - Uniform acceleration and retardation. - Cycloidal motion. |  |  |  |  |  |
| 13 | List the different types of followers. |  | Remember BT-L1 | CO3 |  | PI 1.2.1 |
|  | The different types of followers are - Knife edge follower & Roller follower - Mushroom or flat faced follower - Spherical faced or curved-shoe follower. |  |  |  |  |  |
| 14 | Define trace point of a cam. |  | Understand BT-L2 | CO3 |  | PI 1.4.1 |
|  | It is a reference point on the follower and is used to generate the pitch curve. In case of knife edge follower, the knife edge represents the trace point and the pitch curve corresponds to the cam profile. In a roller follower, the centre of the roller represents the trace point. |  |  |  |  |  |
| 15 | What is meant by undercutting in case of cams? |  | Remember BT-L1 | CO3 |  | PI 1.2.1 |
|  | If the curvature of the pitch curve is too sharp, then the part of the cam shape would be lost and thereafter the intended cam motion would not be achieved. Such a process is said to be undercut of cams. |  |  |  |  |  |
| 16 | What are the Constraints on the Follower? |  | Remember BT-L1 | CO3 |  | PI 1.3.1 |
|  | - Gravity constraint - Spring constraint - Positive mechanical constraint |  |  |  |  |  |
| 17 | What do you know about nomogram? |  | Remember BT-L1 | CO3 |  | PI 1.3.1 |
|  | In nomogram, by knowing the values of total lift of the follower and the cam rotation angle for each segment of the displacement diagram, we can read directly the maximum pressure angle occurring in the segment for a particular choice of prime circle radius. |  |  |  |  |  |
| 18 | Define tangent cam. |  | Understand BT-L2 | CO3 |  | PI 1.3.1 |
|  | The flanks of the cam are straight and tangential to the base circle and nose circle, and then the cam is known as a tangent cam. These cams are usually symmetrical about the centre line of the cam shaft. Such types of cams are used for operating the inlet and exhaust valves of internal combustion engines. |  |  |  |  |  |
| 19 | Define angle of dwell. |  | Remember BT-L1 | CO3 |  | PI 1.2.1 |
|  | Angle of dwell is angle through which the cam rotates while the follower remains stationary at the highest or the lowest position or The period during which the follower remains at rest is called dwell period. |  |  |  |  |  |
| 20 | What is pressure angle in cam? |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | It is the angle between the direction of the follower motion and a normal to the pitch curve. This angle is very important in cam design as it represents steepness of the cam profile. If the pressure angle is too large, a reciprocating follower will jam in its bearings. |  |  |  |  |  |
| 21 | Define undercutting in cam. How it occurs? |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | The cam profile must be continuous curve without any loop. If the curvature of the pitch curve is too sharp, then the part of the cam shape would be lost and thereafter the intended cam motion would not be achieved. Such a cam is said to be undercut. Undercutting occurs in the cam because of attempting to achieve too great a follower lift with very small cam rotation with a smaller cam. |  |  |  |  |  |
| 22 | Define pitch curve of the cam. |  | Remember BT-L1 | CO3 |  | PI 1.2.1 |
|  | It is the curve generated by the trace point as the follower moves relative to the cam. For a knife edge follower the pitch curve and the cam profile are the same, where as for the roller follower, they are separated by the radius of the roller so it is called pitch curve of the cam. |  |  |  |  |  |
| 23 | How can you prevent undercutting in cam? |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | The Prevent undercutting in cam are followed: - By decreasing the follower lift - By increasing cam rotation angle - By increasing the cam size |  |  |  |  |  |
| 24 | Define trace point in the study of cams. |  | Remember BT-L1 | CO3 |  | PI 1.4.1 |
|  | It is a reference point on the follower and is used to generate the pitch curve. In case of knife-edge follower the knife edge represents the trace point and the pitch curve corresponds to the cam profile. In a roller follower the center of the roller represents the trace point. |  |  |  |  |  |

| 25 | Draw Displacement, velocity diagram when the follower moves with Simple Harmonic motion. |  | Create BT-L6 | CO3 | PI 1.4.1 |
| --- | --- | --- | --- | --- | --- |
| PART – B |  |  |  |  |  |
| 1 | The following data relate to a cam profile in which the following is a knife edge follower and moves with SHM during the accent and decent. Minimum radius of cam =25 mm, lift =30 mm, angle of accent =120˚,angle of descent =100˚, angle of dwell between ascent and descent =80˚,speed of the cam = 200 rpm. Draw profile of the cam and determine the maximum velocity and maximum acceleration during outstroke and the return stroke. | AU-ND 2019 | Apply BT-L3 | CO3 | PI 1.4.1 |
| 2 | Why cycloidal cams are suitable for high speed applications? | AU-ND 2019 | Apply BT-L3 | CO3 | PI 1.3.1 |
| 3 | A radial cam, operating a roller follower, rotates at 200 rpm. The follower rises through 20 mm with S.H.M. during 120° of cam rotation. it dwells for 30° of cam rotation and returns to the initial position by S.H.M. in next 150° of cam rotation. Assuming a minimum radius of cam to be 25 mm, and roller diameter as 10 mm draw the cam profile. Determine V max and Amax during outstroke | AU-ND 2019 | Apply BT-L3 | CO3 | PI 1.4.1 |
| 4 | It is required to set out the profile of a cam to give the following motion to the reciprocating follower with a flat mushroom contact face. - Follower to have a stroke of 20 mm during 120° of cam rotation. - Follower to dwell for 30° of cam rotation. - Follower to return to its initial position during 120° of cam rotation; and Follower to dwell for remaining 90° of cam rotation. The minimum radius of the cam is 25 mm. the motion of the follower is to take place with simple harmonic motion during out stoke and return stroke. | AU-AM 2019 | Apply BT-L3 | CO3 | PI 1.4.1 |
| 5 | A flat ended valve tappet is operated ·by a symmetrical cam with circular arc for flank and nose. The straight line. path of the tappet passes through the cam axis. Total angle of action 150°, lift 6mm, base circles diameter 30mm, period of acceleration is the half the period of retardation during the lift. The cam rotates at 1250 rpm. Find (i) Flank and nose radii (ii) Maximum acceleration and retardation during the lift. | AU-AM 2019 | Create BT-L6 | CO3 | PI 1.4.1 |
| 6 | Neatly sketch a cam mechanism with roller follower and indicate the following in the sketch and brief them: Cam profile, based circle, prime circle and pressure angle. |  | Create BT-L6 | CO3 | PI 1.3.1 |
|  | ii) In a cam follower mechanism, 40 mm lift of the follower has to be made in the first 1200 rotation of the cam. Draw the displacement diagrams for the following types of motions, separately for each taking at least 8 equal divisions of 1200 : a) Simple harmonic motion b) Cycloidal motions. |  | Apply BT-L3 | CO3 | PI 1.3.1 |
| 7 | Draw the cam profile of an off set knife edge follower cam, which rotates in clock wise direction with both rise and return have uniform acceleration and retardation mention the following data: Base circle diameter of the cam=50 mm; Lift of the follower=48 mm; Offset of follower=10 mm to the right of cam rotation at centre. Cam rotation angles for the follower motions are: Rise=800, Return=1200 and second dwell=600. Assume the length of the displacement diagram as 180 mm (X-axis) and derive the rise and return rectangles in to 8 equal divisions each. |  | Create BT-L6 | CO3 | PI 1.4.1 |
| 8 | A symmetrical cam operates a flat-footed follower. The lift is 10 mm, base circle radius 20 mm. The total angle of the cam action is 1620. Find the radius of convex flanks and nose and determine the maximum acceleration and retardation during lift when the cam shaft rotates at 1200 rpm Period of acceleration is half the period of retardation during the lift. |  | Apply BT-L3 | CO3 | PI 1.4.1 |
| 9 | Design the cam for operating the exhaust valve of an oil engine. It is required to give equal uniform acceleration and retardation during opening and closing of the valve each of which corresponds to 600 of cam rotation. The valve must remain in the fully open position for 200 of the cam rotation. The lift of the valve is 37.5 mm and the least radius of the cam is 40 mm. The follower is provided with a roller of radius 20 mm and its line of stroke passes through the axis of the cam. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| 10 | Draw the profile of a cam operating a knife edge follower having a lift of 30 mm. The cam rises the follower with SHM for 1500 of the rotation followed by the period of dwell for 600. The follower descends for the next 1000 rotation of the cam with uniform velocity, again followed by a dwell period. The cam rotates at a uniform velocity, again followed by a dwell period. The cam rotates at a uniform velocity of 120 rpm and has a least radius of 20 mm. What will be the maximum velocity and acceleration of the follower during the lift and the return? |  | Create BT-L6 | CO3 | PI 1.3.1 |

| 11 | In a asymmetrical tangent cam operating a roller follower, the least radius of the cam is 30 mm and the roller radius is 17.5 mm. The angle of ascent is 75o and the total lift is 17.5 mm. The speed of the cam shaft is 600 rpm. Calculate: (i) The principal dimensions of the cam; (ii) the accelerations of the follower at the beginning of the lift, where straight flank merges into the circular nose and at the apex of the circular nose; (iii) Draw the profile of the cam. Assume that there is no dwell between ascent and descent. | Create BT-L6 | CO3 | PI 1.4.1 |
| --- | --- | --- | --- | --- |
| 12 | A cam is to be used for a platform that will repeatedly lift boxes from a lower conveyor to an upper conveyor. Plot a displacement diagram and determine the required speed of the cam when the follower motion is as follows: - Rise 40mm in 1.2 s. - Dwell for 0.3 s. - Fall 20 mm in 0.9 s. - Dwell 0.6 s. - Fall 20 mm in 0.9 s. | Create BT-L6 | CO3 | PI 1.4.1 |
| 13 | Draw the displacement, velocity and acceleration curves, when the follower moves with simple harmonic motion and derive the expression for maximum velocity and maximum acceleration. | Create BT-L6 | CO3 | PI 1.2.1 |
| 14 | Follower type = roller follower, lift = 25 mm; base circlr radius = 20 mm roller radius = 5 mm; out stroke with SHM, for 1200 cam rotation; dwell for 600 cam rotation; return stroke with SHM, for 900 cam rotation; dwell for the remaining period. Determine maximum velocity and acceleration during out stroke and return stroke if the cam rotates at 1200 rpm in counter clockwise direction. Draw the cam profile for conditions with follower offset to right of the cam centre by 5 mm. | Create BT-L6 | CO3 | PI 1.2.1 |
| 15 | A disc cam used for moving a knife edge follower with SHM during lift and uniform acceleration and retardation motion during return. Cam rotates at 300 rpm clockwise direction. The line of motion of the follower has an offset 10 mm to the right angle of cam shaft axis. The minimum radius of the cam is 30 mm. The lift of the follower is 40 mm. The cam rotation angles are: lift 600, dwell 900, return 1200 and remaining angle for dwell. Draw the cam profile and determine the maximum velocity and acceleration during the lift and return. | Create BT-L6 | CO3 | PI 1.2.1 |
| 16 | Design a cam to raise a valve with simple harmonic motion through 15mm is 1/3rd of a revolution; keep it fully raised through 1/12th of a revolution and to lower it with SHM in 1/6th of a revolution. The valve remains closed during the rest of the revolution. The diameter of the roller is 20 mm and the minimum radius of the cam is 25 mm. The axis of the valve rod passes through the axis of the cam shaft. If the cam shaft rotates at uniform speed of 100 rpm; find the maximum velocity and acceleration of the valve during raising and lowering. Also draw the profile of the cam. | Create BT-L6 | CO3 | PI 1.2.1 |

| 17 | Layout the profile of a cam operating a roller reciprocating follower for the following data: Lift of follower = 30 mm, Angle during the follower rise period = 120o, Angle during the follower after rise = 30o, Angle during the follower return period = 150o, Angle during which follower dwell after return = 60o, Minimum radius of cam = 25 mm, Roller diameter = 10 mm. The motion of follower is uniform acceleration and deceleration during the rise and return period. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| --- | --- | --- | --- | --- | --- |
| 18 | A cam with minimum radius of 25 mm, rotating clockwise at a uniform speed is to be designed for a roller follower at the end of valve rod. The valve rod is raised through 50 mm during 120o rotation. The angle of dwell is 30o. The valve is lowered through next 600. The valve is closed during the rest of revolution. The diameter of roller is 30 mm and the diameter of cam shaft is 25 mm. Draw: a) Displacement diagram, b)Cam profile. The line of the stroke is offset 15 mm from the axis of the cam shaft. The displacement takes place with SHM. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| 19 | A cam is to give the following motion to a roller follower: Outstroke during 60o of cam rotation (with uniform velocity), dwell for the next 30o of cam rotation, return stroke during next 60o (with uniform velocity) and dwell for the remaining 210o of cam rotation. Draw the velocity and acceleration profiles for a stroke of 40 mm and the minimum radius of the cam is 50 mm. Roller diameter is 10 mm. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| 20 | A pinion having 30 teeth drives a gear having 80 teeth. The profile of the gears is in volute with 20o pressure angle, 12 mm module and 10 mm addendum. Find the length of path of contact, arc of contact and contact ratio. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| PART – C |  |  |  |  |  |
| 1 | In a symmetrical tangent cam operating a roller follower, the least radius of the cam is 30 mm and roller radius is 17.5 mm. The angle of ascent 1s 75° and the total lift is 17.5 mm. The speed of the cam shaft is 600 rpm. Calculate (i) the principal dimensions of the cam (ii) the accelerations of the follower at the beginning of the lift, where straight flank merges into the circular nose and at the apex of the circular nose. Assume that there is no dwell between ascent and descent. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| 2 | For a high speed application using cam mechanism, the motion of the follower should be gradually changing smooth acceleration and with constant pressure angle for whole cycle of rotation of the cam. Suggest a suitable type of motion and a suitable follower, for this application stating the reasons. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| 3 | Draw the profile of a cam operating a roller reciprocating follower and with the following data: Minimum radius of cam =25 mm; lift=30mm; Roller diameter= 15mm. The cam lifts the follower for 120° with SHM, followed by a dwell period of 30. Then the follower lowers down during 150° of cam rotation with uniform acceleration and retardation followed by a dwell period. If the cam rotates at a uniform speed of 150 RPM. Calculate the maximum velocity and acceleration of follower during the descent period. |  | Create BT-L6 | CO3 | PI 1.2.1 |
| UNIT-IV GEARS AND GEAR TRAINS PART – A |  |  |  |  |  |
| 1 | What do you mean by spiral gears and hypoid gears? | AU-ND 2019 | Remember BT-L1 | CO4 | PI 1.3.1 |
|  | A spiral bevel gear is a bevel gear with helical teeth. The main application of this is in a vehicle. A spiral bevel gear set should always be replaced in pairs i.e. both the left. One of a pair of bevel gears that are used especially in automotive transmissions, are designed so that the axis of the pinion does not intersect the axis of the gear, and have the teeth on the pinion cut spirally and the teeth on the gear cut nonradially. |  |  |  |  |
| 2 | Distinguish cycloid and involute profiles of gear tooth. | AU-ND 2019 | Understand BT-L2 | CO4 | PI 1.2.1 |
|  | Involute Gear Profile: - Pressure angle remains same throughout the operation. - Teeths are weaker. - It is easier to manufacture due to convex surface. Cycloid Gear Profile: - Teeths are stronger. - It is difficult to manufacture due to requirement of hypocycloid and epicycloids.. - The centre distance should remain the same. |  |  |  |  |
| 3 | Define module of gears and its relation to circular pitch. | AU-AM 2019 | Remember BT-L1 | CO4 | PI 1.3.1 |
|  | “Module” is the unit of size that indicates how big or small a gear is. The module of a gear, defined as the ratio of the circular pitch by the number π, is the measure of the gear as V. |  |  |  |  |
| 4 | Write the advantages of cycloidal gears. | AU-AM 2019 | Remember BT-L1 | CO4 | PI 1.4.1 |
|  | The advantages of the Cycloidal gears are as follows : - Having a wider flank as compared to Involute gears they are considered to have more strength and hence can withstand further load and stress. - The contact in case of cycloidal gears is between the concave surface and the convex flank. |  |  |  |  |
| 5 | State the two important similarities of a spur gear pair and helical gear pair. |  | Remember BT-L1 | CO4 | PI 1.4.1 |
|  | The two important similarities of a spur gear pair and helical gear pair are 1.Both Spur and Helical geras are transmit power between the parallel shafts. 2. Both Spur and Helical gear designs are simple. |  |  |  |  |
| 6 | Sketch an ordinary gear train and an epicyclic gear train stating their important difference. |  | Create BT-L6 | CO4 | PI 1.4.1 |
|  | Difference of simple and epicyclic gear trains are - a simple gear train, some of the gears are connected to the same shaft - In a compound gear train, each gear has its own shaft for rotation. |  |  |  |  |
| 7 | What is meant by crossed belt drive? |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | A belt is a looped strip of flexible material, used to mechanically link two or more rotating shafts. Belts are looped over pulleys. In a two pulley system, the belt can either drive the pulleys in the same direction, or the belt may be crossed, so that the direction of the shafts is opposite. |  |  |  |  |
| 8 | Write the conditions for maximum power transmission by a belt from one pulley to another. |  | Analyze BT-L4 | CO4 | PI 1.4.1 |
|  | Open flat belt drive, When centrifugal tension is neglected, T1 / T2 = eµθ T1 = Tight side tension in Newton (N) T2 = Slack side tension in Newton (N) µ = Coefficient of friction between the pulley and belt materials θ = Angle of contact (or) angle of lap on the smaller pulley |  |  |  |  |
| 9 | State the law of gearing. |  | Remember BT-L1 | CO4 | PI 1.3.1 |
|  | The law of gearing states that for obtaining a constant velocity ratio, at any instant of teeth the common normal at each point of contact should always pass through a pitch point (fixed point), situated on the line joining the centre’s of rotation of the pair of mating gears. |  |  |  |  |
| 10 | What type of gear arrangement is used to transverse the carriage in lathe machine? |  | Remember BT-L1 | CO4 | PI 1.4.1 |
|  | An apron mechanism for a lathe carriage and cross slide comprising, a power driven member operable at a determined speed, a gear train for transmitting the determined speed to a longitudinal feed driving member for the carriage, a gear train for transmitting the determined speed to a transverse feed driving member for the cross slide. |  |  |  |  |
| 11 | Define gear tooth system. |  | Remember BT-L1 | CO4 | PI 1.4.1 |
|  | A tooth system is a standard which specifies the relationship between addendum, dedendum, working depth, tooth thickness apnd pressure angle to attain interchangeability of gears of tooth numbers but of the same pressure angle and pitch |  |  |  |  |
| 12 | What is an Epicyclic gear train? |  | Remember BT-L1 | CO4 | PI 1.3.1 |
|  | Epicyclic gear train is when the gears are arranged in such a manner that one or more gears move upon and around another gear, which forms a rigid structure, then the gear train is known as epicyclic gear train. |  |  |  |  |
| 13 | Define Pressure angle and Module. |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | Pressure angle: It is the angle between the common normal to two gear teeth at the point of contact and the common tangent at the pitch point. It is denoted by Φ. The standard pressure angles are 14 ½0 and 200. Module: It is the ratio of the pitch circle diameter in millimeters to the number of teeth. It is generally denoted by m. mathematically, Module, m = D/T |  |  |  |  |
| 14 | What are the roles of “Idlers” in gear trains? |  | Remember BT-L1 | CO4 | PI 1.3.1 |
|  | The roles of “Idlers” in gear trains are followed - To connect gears where a large centre distance is required. - To obtain the desired direction (i.e., clockwise or anticlockwise) of motion of the driven gear. |  |  |  |  |
| 15 | Write the applications of reverted gear train. |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | The reverted gear trains are used in - Automotive transmissions, - Lathe back gears, - Industrial speed reducers, and - Clocks (where the minute and hour hand shafts are co-axial) |  |  |  |  |
| 16 | Define arc of contact of gears and Backlash. |  | Remember BT-L1 | CO4 | PI 1.4.1 |
|  | It is the path traced by a point on the pitch circle from the beginning to the end of engagement of a given pair of teeth. It is called as arc of contact of gears. Backlash is defined as the difference between the tooth space and the tooth thickness along the pitch circle. Backlash = Tooth space – Tooth thickness. |  |  |  |  |
| 17 | Define interference in gears. |  | Remember BT-L1 | CO4 | PI 1.4.1 |
|  | The phenomenon when the tip of tooth undercuts the root on its mating gear is known as interference in gears and non standard gear teeth means the gear teeth obtained by modifying the standard proportions of gear teeth parameters is known as non standard gear teeth |  |  |  |  |
| 18 | Define the terms path of contact and arc of contact in gears. |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | Path of contact: It is the path traced by the point of contact of two teeth from the beginning to the end of engagement. Arc of contact: It is the path traced by a point on the pitch circle from the beginning to the end of engagement of a given pair of teeth. It is called as arc of contact of gears. |  |  |  |  |
| 19 | Define addendum and dedendum. |  | Remember BT-L1 | CO4 | PI 1.4.1 |
|  | Addendum is defined as the radial distance of a tooth from the pitch circle to the top of the tooth so it is known as addendum Dedendum is defined as the radial distance of a tooth from the pitch circle to the bottom of the tooth so its known as dedendum. |  |  |  |  |
| 20 | Define (a) Module (b) Diameteral Pitch of gears. |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | Module (m):It is the ratio of pitch diameter to the number of teeth on the gear. m = D/T, where D- pitch circle diameter, T – no of teeth , Diametrical Pitch : It is the number of teeth per unit pitch circle. P = T/D, where D- pitch circle diameter, T – no of teeth |  |  |  |  |
| 21 | What is axial pitch of a helical gear? |  | Remember BT-L1 | CO4 | PI 1.3.1 |
|  | It is the distance, parallel to the axis, between similar faces of adjacent teeth. It is same as circular pitch and is therefore denoted by pc. The axial pitch may also be defined as the circular pitch in the plane of rotation or the diameteral plane. |  |  |  |  |
| 22 | List out the applications of epicyclic gear train. |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | The applications of epicyclic gear train are - The differential gear box of an automobile, - Wrist watches, hoists, pulley blocks and Back gear of lathe machine &Aircrafts etc. |  |  |  |  |
| 23 | Define velocity ratio contact ratio. |  | Remember BT-L1 | CO4 | PI 1.3.1 |
|  | Velocity ratio of a simple gear train is defined as the ratio of the angular velocity of the first gear in the train to the angular velocity of the last gear. Contact ratio It is the ratio of the length of arc of contact to the circular pitch is known as contact ratio. The value gives the number of pairs of teeth in contact. |  |  |  |  |
| 24 | Define gear tooth system. |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | A tooth system is a standard which specifies the relationship between addendum, dedendum, working depth, tooth thickness and pressure angle to attain interchangeability of gears of tooth numbers but of the same pressure angle and pitch |  |  |  |  |
| 25 | List out the characteristics of involute action. |  | Remember BT-L1 | CO4 | PI 1.2.1 |
|  | The characteristics of involute action are followed: - Arc of contact. - Length of path of contact. - Contact ratio. |  |  |  |  |
| PART – B |  |  |  |  |  |
| 1 | Calculate (i) length of path of contact (ii) arc of contact (iii) the contact ratio, when a pinion having 23 teeth drive·s a gear having 57 teeth. The profile of the gears is involute with pressure angle 20°, module 8 mm and addendum equal to one module. | AU-ND 2019 | Apply BT-L3 | CO4 | PI 1.4.1 |
| 2 | In a reduction gear shown in Fig. 14 (b), the input S has 24 teeth. P and C constitute a compound planet having 30 and 18 teeth respectively. If all the gears are of same pitch, find the ratio 9f the reduction gear i.e., ratio of speed of gear S to speed of gear D. Assume A to be fixed. | AU-ND 2019 | Apply BT-L3 | CO4 | PI 2.1.2 |
| 3 | A pair of 200 full depth involute spur gears having 30 and 50 teeth respectively of module 4 mm are in mesh. The smaller gear rotates at 1000 r.p.m. Determine: (i)Sliding velocities at engagement and at disengagement of pair of teeth, and(ii) Contact ratio. | AU-AM 2019 | Apply BT-L3 | CO4 | PI 2.1.3 |
| 4 | In an epicyclic gear train, an arm carries two gears A and B having 36 and 45 teeth respectively. If the arm rotates at 150 rpm in the anticlockwise direction about the centre of the gear A which is fixed, determine the speed of gear B. If the gear A instead of being fixed, makes 300rpm in the clockwise direction, what will be the speed of gear B? | AU-AM 2019 | Apply BT-L3 | CO4 | PI 2.1.3 |
| 5 | State the fundamental law of gearing. Prove this law, by considering and neatly sketching two moving curved surfaces in contact. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 6 | Name the two types of tooth profiles satisfying the law of gearing and brief any one of them. |  | Apply BT-L3 | CO4 | PI 2.1.2 |
| 7 | Explain with neat sketches various classification of gear trains. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 8 | Neatly sketch the gear train called as Fergusson’s Paradox. Explain and prove why it is called Paradox, by assuming suitable number of teeth for the gears of this train. |  | Apply BT-L3 | CO4 | PI 2.4.1 |
| 9 | A pinion having 24 teeth drives a gear having 60 teeth. The profile of the gear is involute with 200 pressure angle. 10 mm module and 10 mm addendum. Find the length of path of contact, arc of contact and the contact ratio. |  | Apply BT-L3 | CO4 | PI 2.1.1 |
| 10 | An epicyclic train of gears is arranged as shown in figure. How many revolutions does the arm, to which the pinion P are attached, when S makes 300 rpm counter clockwise and A is stationary. The number of teeth on the gears S and A are 30 and 130 respectively. |  | Apply BT-L3 | CO4 | PI 2.4.1 |
| 11 | The following data relate to the pair of 200 involute gears in mesh: Module = 6 mm, Number of teeth on pinion = 17, Number of teeth on gear = 49, Addendum on pinion and gear wheel = 1 module. Find: (i) The number of pairs of teeth in contact (ii) The angle turned through the pinion and the gear wheel when one pair of teeth is in contact, and (iii) The ratio of sliding to rolling motion when the tip of the tooth on the larger wheel (1) is just making contact, (2) Is just leaving contact with its mating tooth, and (3) is at the pitch point. |  | Apply BT-L3 | CO4 | PI 2.3.2 |
| 12 | An epicyclic gear train consist of 3 gears A, B, and C as shown in fig. the internal gear A has 72 teeth and gear C has 32 external teeth. The gear B meshes with both gear A and gear C and is carried on an arm EF which rotates about the centre of A at 18 rpm. if the gear A is fixed, find the speed of gears B and C. |  | Apply BT-L3 | CO4 | PI 2.3.2 |
| 13 | The cutter of a broaching machine pulled by square threaded screw of 55 mm external diameter and 10 mm pitch. The operating nut takes the axial load of 400 N on a flat surface of 60 mm internal diameter and 90 mm external diameter. If the coefficient of friction is 0.15 for all contact surfaces on the nut, determine the power required to rotate the operating nut, when the cutting speed is 6 m/min. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 14 | Following data is given for a rope pulley transmitting 23.628 kW. Dia of pulley = 40 cm; speed = 110 rpm; angle of groove = 450; angle of lap = 600; coefficient of friction = 0.28; No. of ropes = 10; Mass in kg/m, length of the ropes = 0.0053 x C2 and the working tension is limited 12.2 C2 N where C = grith of rope in cm. Find (i) initial tension and (ii) diameter of each rope. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 15 | Derive an expression for the minimum number of teeth required on the pinion to avoid the interference involute gears. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 16 | Two mating gears have 20 and 40 in volute teeth of module 10 mm and 20o pressure angle. The addendum on each wheel is to be made of such a length that the line of contact on each side of the pitch point has half of the maximum possible length. Determine the addendum height for each gear wheel, length of the path of contact, arc of contact and contact ratio. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 17 | A reverted compound gear train is used as back gear of a lathe. It is required to give a reduction from cone-pulley speed to spindle speed of approximately 9 to 1. The module of the teeth on the high-speed pair is 4 mm and of those on low-speed pair is 5 mm. The centre distance is 180 mm. Determine the number of teeth on each of the four wheels, if the pinions are to have as nearly as possible equal numbers of teeth. Also sketch a line diagram and show the gear train. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 18 | Explain what is meant by interference and undercutting. Discuss various methods used to avoid interference. |  | Apply BT-L3 | CO4 | PI 2.1.2 |
| 19 | An Epicyclic train is composed of fixed annular wheel A having 150 teeth. Meshing with A is wheel B which drives Wheel D through and idle wheel C, wheel D being concentric with A. Wheels B and C are carried on an arm E which revolves clockwise at 100 rpm about the axis of A and D. If the wheel B and D have 25 and 40 teeth respectively, find the number of teeth on C and sense of rotation of C. Also sketch the arrangement. |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| 20 | Two 20o involute spur gears mesh externally to give a velocity ratio of 3. If the module is 3 mm and addendum equal to one module, determine (a) minimum number of teeth on each wheel to avoid interference and (b) the number of pairs of teeth in contact. |  | Apply BT-L3 | CO4 | PI 2.1.2 |
| 21 | The centre distance between two spur gears in mesh is to be approximately 250 mm. The gear ratio is 4:1; the pinion transmits 300kw at 100 rpm. The pressure angle of in volute teeth is 20o and the addendum is equal to one module. The limiting value of normal tooth pressure is 1 KN/mm of width. Determine - Nearest standard module, so that interference does not occur. - Number of teeth on each wheel & Width of the pinion. |  | Apply BT-L3 | CO4 | PI 2.1.3 |

| 22 | (i) State and prove the law of gearing. (ii) Show that the in volute curves as the profiles of mating gears satisfy the law of gearing. |  | Apply BT-L3 | CO4 | PI 1.2.1 |
| --- | --- | --- | --- | --- | --- |
| 23 | A pair of spur gears with in volute teeth is to give a gear ratio of 3:1. The arc of approach is not to be less than the circular pitch and smaller wheel is the driver. The angle of pressure is 20°. (i) What is the least number of teeth that can be used on each wheel? (ii) What is the addendum of the wheel in terms of circular pitch? |  | Apply BT-L3 | CO4 | PI 2.1.3 |
| PART – C |  |  |  |  |  |
| 1 | An epicyclic gear train for an electric motor , is shown in figure. The wheel S has 15 teeth and is fixed to motor shaft rotating at 1450 rpm. The planet P has 45 teeth, gears with fixed annular A and rotates on a spindle carried by an arm which fixed to output shaft. The planet P also gears with the sun when S. Find the speed of output shaft. If motor is transmitting 2 KW find the torque required to fix the annular. |  | Apply BT-L3 | CO4 | PI 2.1.2 |
| 2 | An epicyclic gear train as shown in figure is composed of a fixed annular wheel A having 150 teeth. The wheel A is meshing with wheel B which drives wheel D through an idle wheel C, D being concentric with A. The wheels B and C are carried on an arm which revolves clockwise at 100 rpm about the axis of A and D. If the wheels B and D have 25 and 40 teeth respectively, determine the number of teeth on C and speed and sense of rotation of wheel C. |  | Apply BT-L3 | CO4 | PI 2.1.2 |
| UNIT-V FRICTION IN MACHINE ELEMENT PART – A |  |  |  |  |  |
| 1 | Define lead and pitch of a screw thread. | AU-ND 2019 | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | Pitch is the distance between screw grooves and is commonly used with inch sized products and specified as threads per inch. Lead is the linear travel the nut makes per one screw revolution and is how ball screws are typically specified. |  |  |  |  |
| 2 | State the two assumptions based on which the bearings are designed. | AU-ND 2019 | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | While it might be a bit of a leap, I’m going to assume that the bearing is well-designed, well-manufactured, properly handled and stored, and finally, correctly selected for the intended application. With that said, we’re now ready to talk about those silent assumptions that are in the maintenance function’s domain. |  |  |  |  |
| 3 | What is the effect of centrifugal tension in belt drives? | AU-AM 2019 | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | This centrifugal force reduces the pressure between the belt and the rim of the pulleys, and thus, moves the belt away from the pulley and reduces the wrap angle. Therefore, the belt tensions and power transmission also are reduced. Well, generally the belt speed is directly proportional to the power transmission. |  |  |  |  |
| 4 | What are the advantages of hydraulic brake over other brakes? | AU-AM 2019 | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | - Hydraulic brakes have advantages over traditional brakes are, - Hydraulic brakes are more efficient than most brakes when coming to a complete stop. - Hydraulics also offer better cost economically and space compared to other types of brakes. |  |  |  |  |
| 5 | In an open belt drive of horizontal type, the slack side of belt should be kept on the top side of pulleys. Why? |  | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | In an open belt drive of horizontal type, the slack side of belt should be kept on the top side of pulleys because of the angle wrap increase which in turn increases the contact area between belt and pulley, therefore friction increases and slip of belt will be minimum. |  |  |  |  |
| 6 | What are the advantages of using friction clutches? |  | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | The advantages of using friction clutches are - Smooth engagement and minimum shock during the engagement - Friction clutch can be engaged and disengaged when the machine is running since they have no jaw or teeth. - Easy to operate. They are capable of transmitting partial power. |  |  |  |  |
| 7 | Give the classification of gears based on position of teeth on the wheel. |  | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | According to position of teeth on the gear surface, the teeth on the gear surface may be straight, inclined or curved. The spur gears have straight teeth where as helical gears have their teeth inclined to the wheel trim. In case of spiral gears, the teeth are curved over the rim surface. |  |  |  |  |

| 8 | Draw the compound gear train. |  | Create BT-L6 | CO5 | PI 1.3.1 |
| --- | --- | --- | --- | --- | --- |
| 9 | What kind of friction acts between tyre & road in an automobile? |  | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | If the tyres are rolling without slipping, the friction that comes into play is static friction. This is because the velocity of the part of tyre in contact with the road is zero relative to the road. However, this is true only for pure rolling friction. |  |  |  |  |
| 10 | State the functional difference between a clutch and a brake. |  | Remember BT-L1 | CO5 | PI 1.2.1 |
|  | The functional difference between a clutch and a brake are, - A clutch connects two moving members of a machine, Whereas a brake connects a moving member to a stationary member. |  |  |  |  |
| 11 | What is the function of a clutch in automobiles? |  | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | The function of a clutch in automobiles are - It supplies power to the transmission system. - It stops the vehicle by disconnecting the engine from the transmission system. - It is used to change the gear and idling the engine. It gives gradual increment of speed to the wheels. |  |  |  |  |
| 12 | Differentiate between self locking and overhauling of screw. |  | Remember BT-L1 | CO5 | PI 1.2.1 |
|  | The load will start moving downward without the application of any torque is called as overhauling of screws. The torque required to lower the load, indicating that an effort is applied to lower the load is known as self locking screws. |  |  |  |  |
| 13 | Why self locking screws have lesser efficiency? |  | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | For a self locking screw, friction angle Φ must be greater than helix angle α. The efficiency of self-locking screws is less than 50%. If the efficiency is more than 50% then the screw is said to be overhauling. |  |  |  |  |
| 14 | What is meant by self-locking and a self-energized brake? |  | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | Self-locking Brake: When the frictional force is sufficient enough to apply the brake is said to be self-locking brake. Self-energizing Brake: The moment of applied force and the moment of the frictional force about are in same direction. Thus frictional force helps in applying brake is called as self-energized brake. |  |  |  |  |
| 15 | Define friction clutches and dry friction. |  | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | It is a transmission device of an automobile which is used to engage and disengage the power from the engine to the rest of the system. Dry friction is defined as The friction that exists between two un lubricated surfaces is known as dry friction. |  |  |  |  |
| 16 | State the law of belting. |  | Remember BT-L1 | CO5 | PI 1.2.1 |
|  | Law of belting states that the centre line of the belt, as it approaches the pulley must lie in a plane perpendicular to the axis of that pulley or must lie in the plane of the pulley, otherwise the belt will run off the pulley. |  |  |  |  |
| 17 | Mention the materials used for belt. |  | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | Belt Types; Belt Materials. Flat belts; Leather canvas, cotton and rubber. V-belts; Rubberized fabric and rubber. Ropes; Cotton, hemp and manila.. |
| 18 | What are the classifications of friction? |  | Remember BT-L1 | CO5 | PI 1.2.1 |
|  | The classifications of friction are - Skin or Greasy friction. - Film friction and Dry friction or coulomb friction. |  |  |  |  |
| 19 | What are the functions of clutches? |  | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | The functions of clutches are - It supplies power to the transmission system. - It stops the vehicle by disconnecting the engine from transmission system. - It is used to change the gear and idling the engine. It gives gradual increment of speed to the wheels. |  |  |  |  |
| 20 | Where is a V belt used? |  | Remember BT-L1 | CO5 | PI 1.2.1 |
|  | The V belt is mostly used in the factories and workshops where a great amount of power is to be transmitted from one pulley to another when the two pulleys are very near to each other so these are the uses of V belt. |  |  |  |  |
| 21 | List out any four desirable characteristics of brake lining material. |  | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | The desirable characteristics of brake lining material are - Low wear rate - High heat resistant - It should have high coefficient of friction with minimum fading It should have adequate mechanical strength and high heat dissipation capacity |  |  |  |  |
| 22 | List out the commonly used breaks. |  | Remember BT-L1 | CO5 | PI 1.2.1 |
|  | The commonly used Breaks are - Hydraulic brakes: e.g., Pumps or hydrodynamic brake and fluid agitator. - Electric brakes: e.g., Eddy current brakes and Mechanical brakes: e.g., Radial and axial brakes. |  |  |  |  |
| 23 | What is meant by initial tension in belts? |  | Remember BT-L1 | CO5 | PI 1.3.1 |
|  | In order to increase the frictional grip between the belt and pulleys, the belt is tightened up. Due to this belt gets subjected to some tension even when the pulleys are stationary. This tension in the belt is called initial tension (T0). |  |  |  |  |
| 24 | Why lubrication reduces friction? |  | Remember BT-L1 | CO5 | PI 1.2.1 |
|  | In practical all the matting surfaces are having roughness with it. It causes friction. If the surfaces are smooth then friction is very less. Lubrication smoothens the matting surface by introducing oil film between it. The fluids are having high smoothness then solids and thus lubrication reduces friction. |  |  |  |  |
| 25 | When is the cross belt used instead of open belt? |  | Remember BT-L1 | CO5 | PI 1.4.1 |
|  | - Cross belt is used where the direction of rotation of driven pulley is opposite to driving pulley so at the time cross belt is used instead of open belt. - Where we need more power transmission there we can use cross belt drive. |  |  |  |  |
| PART – B |  |  |  |  |  |
| 1 | An open belt running over two 1Pulleys 240 mm and 600 mm diameter connects two parallel shafts 3 metres apart and transmits 4 kW from the smaller pulley that rotates at 100 r.p .m. Coefficient of friction between the belt and the pulley is 0.3 and the safe working tension is 10 N per mm width. Determine - Minimum width of the belt. - Initial belt tension, and - Length of the belt required | AU-AM 2019 | Apply BT-L3 | CO5 | PI 2.4.1 |
| 2 | A dry single plate clutch is .to be designed for an automotive vehicle whose engine· is rated to give 100 kW at 2400 r.p.m. and maximum torque 500 N-m. The outer radius of friction plate is 25% more than the inner radius. The intensity of pressure between the plate is not to exceed 0.07 N/mm2. The coefficient of friction may be assumed equal to 0.3. The helical springs required by the clutch to provide axial force necessary to engage the clutch are eight. If each spring has stiffness equal to 40 N/mm, determine the initial compression in the springs and dimensions of the friction plate. | AU-AM 2019 | Analyze BT-L4 | CO5 | PI 2.4.1 |
| 3 | A double start square thread screw with 50 mm major diameter has 8 mm pitch. The coefficient of friction between screw and nut is 0, 1. If the nut is held fixed, determine the torque required on the screw to raise and to lower a load of 40 kN assuming the load to rotate with the screw. State giving reasons whether the sci1ew is self-locking or over hauling. | AU-AM 2019 | Apply BT-L3 | CO5 | PI 2.4.1 |
| 4 | State and prove the relationship between angle of friction and co-efficient of friction with suitable sketches. |  | Remember BT-L1 | CO5 | PI 2.1.2 |

| 5 | An open belt running over two pulleys of diameters 600 mm and 200 mm connects two parallel shafts which are 2.5 m apart. The smaller pulley transmits 7.5 KW at 300 rpm. The co-efficient of friction between the pulley and the belt is 0.3. Determine the ratio of tension on tight side, T1 with tension on slack side, T2 and the initial tension on the belt. | Apply BT-L3 | CO5 | PI 2.4.1 |
| --- | --- | --- | --- | --- |
| 6 | Neatly sketch a simple Band Brake and derive the equation s for braking torque for both directions of rotation separately and compare them. | Apply BT-L3 | CO5 | PI 2.1.3 |
| 7 | The outer and inner radii of a flat collar thrust bearing are 120 mm and 72 mm respectively. The total axial thrust is 60 KN and the intensity of uniform pressure is 0.25 MPa. If the coefficient friction is 0.05 and the shaft rotates at 600 rpm, determine the power lost in overcoming the friction and the number of collars required to withstand the axial thrust. | Apply BT-L3 | CO5 | PI 2.1.3 |
| 8 | A cross belt running over two pulleys 600 mm and 300 mm diameter connects two parallel shafts 4 meters apart and transmits 7.5 kW from the larger pulley that rotates at 225 rpm. Coefficient of friction between the belt and pulley is 0.35 and the safe working tension is 25 N per mm width. Determine 1. Minimum width of the belt 2. Initial belt tension and 3. Length of the belt required. | Analyze BT-L4 | CO5 | PI 2.1.2 |
| 9 | An electric motor driven power screw moves a nut in a horizontal plane against a force of 75 kN at a speed of 300 mm/min. The screw has a single square thread of 6 mm pitch on a major diameter of 40 mm. The coefficient of friction at the screw threads is 0.1. Estimate power of the motor | Analyze BT-L4 | CO5 | PI 2.1.3 |
| 10 | The following data relate to a screw jack: Pitch of the threaded screw = 8 mm, Diameter of the threaded screw = 40 mm, Coefficient of friction between screw and nut = 0.1, Load = 20 KN. Assuming that the load rotates with the screw, determine the Ratio of torques required to raise and lower the load Efficiency of the machine. | Apply BT-L3 | CO5 | PI 2.1.3 |
| 11 | A single plate clutch transmits 25 kw at 900 rpm. The maximum pressure intensity between the plates is 85 kN/m2. The outer diameter of the plate is 360 mm. Both the sides of the plate are effective and the coefficient of friction is 0.25. Determine the - Inner radius of the plate - Axial force to engage the clutch. | Analyze BT-L4 | CO5 | PI 2.1.2 |
| 12 | Explain gear nomenclature with neat diagram and define all salient terms pertaining to the gear. | Remember BT-L1 | CO5 | PI 2.1.3 |

| 13 | Figure shows a differential gear used in a motor car. The pinion A on the propeller shaft has 12 teeth and gears with the crown gear B which has 60 teeth. The shafts P and Q form the rear axles to which the road wheels are attached. If the propeller shaft rotates at 1000 rpm and the road wheel attached to the axle Q has a speed of 210 rpm. While taking a turn, find the speed of road wheel attached to axle P. | Apply BT-L3 | CO5 | PI 2.2.1 |
| --- | --- | --- | --- | --- |
| 14 | A single plate clutch, with both sides effective, has outer and inner diameters 300 mm and 200 mm respectively. The maximum intensity of pressure at any point in the contact surface is not to exceed 0.1 N/mm2. If the coefficient of friction is 0.3, determine the power transmitted by a clutch at a speed 2500 rpm for two types of assumptions, that is, for uniform pressure and for uniform wear. | Analyze BT-L4 | CO5 | PI 2.1.2 |
| 15 | Two pulleys, one 450 mm diameter and the other 200 mm diameter are on parallel shafts 2.1 m apart and are connected by a belt, as a cross belt drive. The larger pulley rotates at 225 rpm. The maximum permissible tension in the belt is 1 KN and the coefficient of friction between the belt and the pulley is 0.25. Find the power that can be transmitted. | Apply BT-L3 | CO5 | PI 2.1.2 |
| 16 | The following data related to a screw jack; Pitch of the thread screw = 8 mm, diameter of the screw thread = 40 mm, Coefficient of friction between screw and nut = 0.1, load = 20KN. Assuming that the load rotates with screw, determine: a) The ratio of torques required to raise and lower the load. b) The efficiency of the machine. | Apply BT-L3 | CO5 | PI 2.1.3 |
| 17 | In a simple band brake, one end of the band is attached to the fulcrum of a lever. The other end is attached at a distance of ‘b’ from the fulcrum. The effort is applied at the end of the lever. Derive an expression for braking torque, in terms of the effort. | Understand BT-L2 | CO5 | PI 2.1.2 |
| 18 | A weight of 20 KN is lifted by a screw jack of having 10 mm pitch and 60 mm mean diameter. Determine efficiency of the screw jack when - The weight rotates with the screw, and - The load rest on a collar which does not rotate with the screw. The external and internal diameters of the collar surface are 80 mm and 20 mm respectively. The screw and collar friction are 0.08 and 0.1 respectively. | Analyze BT-L4 | CO5 | PI 2.1.3 |
| 19 | A single plate clutch with both sides effective has inner and outer diameter of 300 mm and 200 mm respectively. The pressure if applied by means of set of springs and limited to 150KN/m2. The speed of clutch is 2800 rpm. Assume the coefficient of friction as 0.3. Determine the power transmitted by the clutch in kW. | Analyze BT-L4 | CO5 | PI 3.1.3 |

| 20 | A flat belt drives a pulley with an angle of lap 1200. The belt with a cross section of 100 mm x 60 mm and density of 1000 Kg/m3 has a coefficient of friction 0.3. If the maximum permissible stress in the belt is 2 N/mm2, find the greatest power which can be transmitted and the speed at which it can be transmitted | Analyze BT-L4 | CO5 | PI 3.1.3 |
| --- | --- | --- | --- | --- |
| 21 | The following data refer to a car in which brakes are applied to the front wheels, wheel base= 2.8 m; center of mass from rear axle = 1.3 m; centre of mass above level is 0.96 m; coefficient of friction between the wheel and tyres is 0.4. If the speed of the car is 50 Km/hr, find the distance travelled by the car before coming to rest when the car moves (a) up an incline 1 in 16 (b) down an incline 1 in 16 and (c) on a level track. | Apply BT-L3 | CO5 | PI 3.2.2 |
| 22 | A thrust bearing of a propeller shaft consists of a number of collars. The shaft is of 500 mm diameter and rotates at a speed of 100 rpm. The thrust on the shaft is 300 KN. If the intensity pressure is limited to 250 KN/m2 and coefficient of friction is 0.05. Determine the external diameter and number of collars. The power lost to friction is limited to 50KN. | Apply BT-L3 | CO5 | PI 1.4.1 |
| PART-C |  |  |  |  |
| 1 | Data related to square Threaded Screw Jack are: Pitch diameter=60 mm, Pitch thread=16 mm, Load=30 KN, Co efficient of friction between screw and nut is 0.2. Determine the ratio of torque required to lower and to raise the load. Also, find the efficiency of the screw jack when the load is raised. | Analyze BT-L4 | CO5 | PI 2.3.1 |
| 2 | A belt drive consists of two V belts in parallel on grooved pulleys of the same size. The angle of the groove is 30°. The cross sectional area of each belt is 750mm2 and μ=0.12.The density of the belt material is 1.2 Mg/m3 and the maximum safe stress in the material is 7 MPa. Calculate the power that can be transmitted between the pulleys 300mm diameter rotating at 1500 rpm. Find also the shaft speed in rpm at which the power transmitted would be maximum. | Evaluate BT-L5 | CO5 | PI 3.1.4 |